#### LED 40*10W Waterproof Light

- Graphic section:

A. function key

B. additional key

C. The key reduction

D. Confirmation key

E. Digital display window

A    B    C   D

Instructions: Press function key A to cycle different functions, and press key B or C to modify its parameter value. Press the key D to confirm.

- Operating instructions of the four-digit digital button board

| order number | menu | Menu function | explain |
| --- | --- | --- | --- |
| 1 | d001 | DMX512 Address code setting 001 | Press upper and down keys to adjust the address code, press OK to save. |
| 2 | r255 | Red brightness regulation 255 | Press the upper and lower key to adjust the red brightness, and press the OK key to save. |
| 3 | G 255 | Green brightness adjustment 255 | Press the upper and lower keys to adjust the green brightness, and press the OK key to save. |
| 4 | b255 | Blue brightness regulation 255 | Press the upper and down keys to adjust the blue brightness, press the OK key to save. |
| 5 | H255 | White brightness regulation 255 | Press the upper and lower keys to adjust the white brightness, and press the OK key to save. |
| 6 | P r01 | Built-in program setting 01 | Press the up and down keys to select the built-in program, and press OK to save. |
| 7 | S P01 | Built-in program speed setting 16 | Press up and down keys to select the built-in program speed, and press OK to save. |
| 8 | LD01 | Selection of dimming mode | Press up and down to select the Smooth dimming or fast dimming mode. |
| 9 | AUTO | Since go | Press OK to save. |
| 10 | AUDI | sound control | Press OK to save. |
| 11 | N T00 | Displays the current lamp-bead operating temperature | Current operating temperature of the lamp bead |
| 12 | EP00/01 | Signals remain selected EP00 takes hold, EP01 does not. | Press up and down to select the signal to hold or not, press OK to save. |
| 13 | DP00/01 | The setting button is not operated for 30 seconds, and the digital tube is not bright. D P01 is always bright. | Press the up and down keys to determine whether to always bright. Press OK to save. |
| 14 | DISP | Show inverted selection | Press the up and down keys to select the positive and reverse displays. |

3.DMX512 channel description

| order number | function | explain |
| --- | --- | --- |
| Channel 1 | Total dimming | (0-255) R, G, B, W total dimming, linear dimming, from dark to bright |
| Channel 2 | Total frequency flash | (0-255) R, G, B, W, from slow to fast |
| Channel 3 | R aiming | R dimming, (0-255) Linear dimming, from dark to bright |
| Channel 4 | G aiming | G dimming, (0-255) Linear dimming, from dark to bright |
| Channel 5 | B aiming | B dimming, (0-255) Linear dimming, from dark to bright |
| Channel 6 | W aiming | W dimming, (0-255) Linear dimming, from dark to bright |
| Channel 7 | shade selection | The combination of colors. |
| Channel 8 | macro | (0-10) Top 7 channel enabled; (11-120) jump; (121-180): gradient; (181-244): jump, gradient; (245-255) voice control |
| Channel 9 | Macro function speed | 0-255 Slow speed, 0 fastest speed. |

In the address code state, after the multi-machine is connected, if there is no connection console, the main and slave online function can be realized to operate one of the devices, and the other devices in the address d001 state can synchronize the program with the host. If the console signal is received after the console is connected, the d of the address code will become A, and the device in the address state can be controlled by the console.